GUIDELINES FOR CONTRACTOR’S QUALITY CONTROL

| Item Description | Property | Minimum Frequency |
| --- | --- | --- |
| 207 Subgrade | Gradation | One (1) sample per day of placement. Note 1 |
|  | Atterburg Limits | From an approved aggregate source: one (1) test at the beginning of placement and then each 10,000 tons. Not from an approved aggregate source a minimum of one (1) test per 6 days placement. |
| 212 select Material for Backfill | Gradation | Minimum of one (1) sample per day of Placement. Note 1 |
| 307 Crushed Aggregate | Gradation | One (1) sample per each one-half (1/2) day placement. Note 1 |
|  | Atterburg limits | One(1) test at the beginning of placement and then each 10,000 tons thereafter |
|  | Other tests as requested by the Division or required by the contract documents: percent crushed particles, unit weight, etc. | As requested by the Division or required by the contract documents. |
| 307 aggregate Shoulder course for Resurfacing Projects | Gradation | One (1) sample per day of placement. Note 1 |
|  | Atterburg limits | One(1) test at the beginning of placement and then each 10,000 tons thereafter |
|  | Other tests as requested by the Division or required by the contract documents: percent crushed particles, unit weight, etc. | As requested by the Division or required by the contract documents. |

GUIDELINES FOR CONTRACTOR’S QUALITY CONTROL

| 604 Class 1 Aggregate | Gradation | Minimum of one (1) sample per day of placement. Note 1 |
| --- | --- | --- |
| 606 Aggregate for Underdrain | Gradation | Minimum of one (1) sample per day of placement. Note 1 |
| 609 Bed Course Material | Gradation | Minimum of one(1) sample per day of placement. Note 1 |
| 626 Aggregate | Gradation | Minimum of one (1) sample per day of placement. Note 1 |
|  | Atterburg Limits | From an approved aggregate source: one (1) test at the beginning of placement and then each 10,000 tons. Not from an approved aggregate source a minimum of one (1) test per 6 days placement. |
| 636 Aggregate | Gradation | One (1) sample per each one-half (1/2) day of placement. Note 1: Note 2 |
|  | Atterburg Limits | One (1) test at beginning of placement and then each 10,000 tons thereafter. Note 2 |

Note 1:	In the event project activities are such that relatively small quantities of material are being placed per 	placement date, and  to prevent over sampling, the Engineer may approve the following alternate 	sampling method : A minimum of One (1) sample per six (6) consecutive days shall be taken to 	represent up to each 170 cubic yards(250 tons).   Sampling is to be done on the first day of aggregate 	placement.  In this case the sample shall be taken at a random time and place

Note 2:	When Aggregate for maintaining traffic is not to be part of any succeeding base or pavement course, 	the appropriate 	aggregate size shall be determined by the Engineer.  If the aggregate is from an 	approved source, then it shall be accepted by visual inspection.  If the Contractor elects to use 	aggregate from an unapproved source, test results shall be provided to show that the liquid limit and 	plasticity index meet the requirements in Table 704.6.2B

***  EXAMPLE GUIDE  FOR AGGREGATE ITEMS QUALITY CONTROL PLAN  ***

The Acme Company

20 First St.

Somewhere, WV  XXXXXXX

Mr/./Ms/Mrs.  _______________________

WV Department of Highways

District  ___   Engineer/Manager

__________,  WV

RE:  “year”    Master Aggregate Items QC Plan

DISTRICT:  _____

Dear Mr./Ms/Mrs. ___________________

We are submitting our Master QC Plan for Aggregate Items , developed in accordance with the (year) WVDOH Standards and Specifications, (year)WVDOH Supplemental specifications, MP300.00.51, MP 700.00.54, ML-25, and AASHTO Testing standards.

The Quality Control Program is under the direction of __________________ .  He/She can be contacted by telephone number _____________,  email __________________ and/or in person.

1.)	All testing will be performed by qualified personnel as per WVDOH Specification Section 	106 Control of Materials.   Proof of personnel certification shall be provided to WVDOH 	inspectors upon request.

2.)	Specify items to be controlled and the methods by which each item will be tested (For 	example:207,307…etc) Attachment #1 summarizes the different materials, minimum 	frequencies, and the appropriate test procedure or method for controlling each material.

- 207 Items	- 212 Items	-307 Crushed Aggregate Items	- ETC>>>>>

3.)	List the location (address) and lab where testing will be performed.

4.)	State the method and means by which that Contractor will document and distribute test 	results.

5.)	State what forms will be used for tests the time frame for completing testing and distributing 	of test information to District Materials.

6.)	Specify in the QC Plan where and how the charts will be maintained and made available to 	Division/District personnel.  Control Charts will use the moving average concept as 	described in MP 300.00.51.

7.)	Specify a plan of action providing for immediate notification of all parties involved in the 	event that nonconforming material situations are detected.

Very Truly Yours,

_____________________________

Title

********   WVDOH  LETTERHEAD   ********

ACME Company

20 First St.

SOMEWHERE, WV  #####

RE:       Aggregate Items Master QC Plan

Description:  (Year)   Construction Season

Dear Sir/Madam,

Your Master Aggregate Quality Control Plan (M#-#####) for 	_________________has been reviewed and found to be acceptable for the 	following 	items:

- 207 Aggregate Items	-    212 Aggregate Items

-      307 Aggregate Items 	-     ETC

As work progresses throughout the season, an addendum(s) may be required to this QCP to keep the QC program current. Also note that personnel may be required to show proof of certification for testing.  Please use Lab Reference # M#-###### when corresponding about this QC plan.  Please make sure that all appropriate personnel have a copy of this plan in their possession.

Very Truly Yours,

_____________________

Title

********  EXAMPLE  ********

THE ACME COMPANY INC.

20 First St.

Somewhere, WV  XXXX

Mr./Ms/Mrs  _______________________

WV Department of Highways

District  ___   Engineer/Manager

__________, WV

Subject:      Aggregate Items QC plan

For project

Fed. Project No _____________________________

State Project No.     __________________________

Contract ID No.      __________________________

Description	  _____________________________

Dear Mr./Ms/Mrs. _____________________,

We would like to use our approved Aggregate Items Master Quality Control Plan,   reference number ______________ for the project referenced above.   We feel that all items on the referenced project are covered by the Master Quality Control Plan for Aggregate Items.

The QC Plan is under the direction of ___________________________________,  ________________(title),  and will be the Company’s contact representative to the Department of Highways  District Materials and Construction Departments.  He/She can be contacted in person at the project, by telephone ________________________________ or at email account _________________.

Very Truly Yours,

______________________

Title

***************   WVDOH  LETTERHEAD   **************

THE ACME COMPANY INC.

20 First St.

Somewhere, WV  XXXXX

RE:  _____ Aggregate Items QC Plan

Project CID#:  ########

Fed/State Project #:   ####- ## - ####.##

Description:  Falling Slide

County :   XXXXXXX

Dear Sir/Madam,

Your request to use your Master Aggregate Items Quality Control Plan (M# - ######) 	for Aggregate 	Items on the project referenced above, has been reviewed and found to 	be acceptable for the following items:

- 207 Aggregate Items	 		- 212 Aggregate Items

- 307 Aggregate Items 		- ETC

As work progresses throughout this project an addendum(s) may be required to this QCP to keep the QC program current.  Please use M# - ###### when corresponding about this QC plan.  Also note that personnel may be required to show proof of certification for testing. Please make sure that all appropriate personnel have a copy of this plan in their possession.

For Division/District use

The Master Quality Control Plan can be reviewed in Projectwise at this Link :

WVDOT ORG>D0#>year>MASTER QC PLANS>Contractors or Plant>Contractor Name>Name of Quality Control Plan

Very Truly Yours,

________________________________

Title