WEST VIRGINIA DEPARTMENT OF TRANSPORTATION

DIVISION OF HIGHWAYS

MATERIALS CONTROL, SOILS AND TESTING DIVISION

MATERIALS PROCEDURE

guide for quality control and acceptance plans for subgrade, base course, and aggregate items

# PURPOSE

The purpose of this Materials Procedure (MP) is to establish minimum requirements for the Contractor’s Quality Control program and Acceptance plan.  It is intended that these requirements be used as a procedural guide in detailing the inspection, sampling, and testing deemed necessary to maintain compliance with the specification requirements.

To establish procedural guidelines for approval and documentation of a Master Quality Control Plan.

# SCOPE

This procedure is applicable to Aggregate items placed in the field.  It outlines the quality control procedures for Aggregate items used in field operations and includes procedures for approving and using a Master and/or Project Specific Quality Control (QC) Plan.  This procedure also aids in documentation and retention of the QC Plan in ProjectWise.

# Refferenced documents

Material Procedures:

- MP 300.00.51,  Procedural  Guidelines  for  Maintaining  Control  charts  for Aggregate Gradations

- MP 700.00.54, Procedure for Evaluating Quality Control Sample Test Results with Verification Sample Test Results

Materials Letter

- ML-25, Procedure for Monitoring the Activities Related to Sieve Analysis of Fine and Coarse Aggregate

- MP 700.00.06  Aggregate Sampling Procedures

# general requirements

The Contractor shall provide and maintain a quality control system that will provide reasonable assurance that all materials and products submitted to the Division for acceptance will conform to the contract requirements whether manufactured or processed by the Contractor or procured from suppliers, subcontractors, or vendors. The contractor shall perform or have performed the inspections and tests required to substantiate product conformance to contract document requirements and shall also perform or have performed all inspections and tests otherwise required by the contract. The Contractor's quality control inspections and tests shall be documented and shall be available for review by the Engineer throughout the life of the contract. The Contractor shall maintain standard equipment and qualified personnel as required by the Specifications to assure conformance to contract requirements. Procedures will be subject to the review of the Division before the work is started.

# Quality control plan

The contractor shall prepare a Quality Control Plan (QC Plan) detailing the type and frequency of inspection, sampling, and testing deemed necessary to measure and control the various properties of materials and construction governed by the Specifications. As a minimum, the sampling and testing plan should detail sampling location, sampling techniques, and test frequency to be utilized. Attachment #2 shows an example QC Plan. Quality control sampling and testing performed by the Contractor may be utilized by the Division for acceptance.

### A QC Plan must be developed by the Contractor and submitted to the Engineer prior to the start of construction on every project. Acceptance of the QC Plan by the Engineer will be contingent upon its concurrence with these guidelines.

As work progresses, an addendum(s) may be required to a QC Plan to keep the QC program current. Personnel may be required to show proof of certification for testing.

Quality Control Plan Guidelines

Name of company official responsible for quality control program.  A phone number and an email address for contacting this individual shall be included in the cover letter.

State your company will provide certified personnel as specified in section 106 of Specifications, either from your company or a consultant testing firm.

List of Aggregate items to be controlled by QC Plan.

Sampling and Testing Plan: As a minimum, the sampling and testing plan should detail sampling locations, test methods, and test frequencies to be used (Attachment 1). To facilitate the Division of Highway’s monitoring activities, which are described in Section 7.1, all completed gradation samples must be retained by the Contractor until further disposition is designated by the District Materials Supervisor. The QC Plan 	should state where and how these samples will be maintained. Applicable sections of Materials Letter (ML) 25 should be used for guidance.

Testing Facility: The plan should state the specific location where the samples(s) will be tested and retained.

Documentation Plan:  The method by which the Contractor will document and distribute test results must be described.

Forms and Distribution: Approved processing forms furnished by the Division will be used to record the test data. Gradation tests will be recorded on Form T300. The laboratory 	reference number will always start with a "C" for all quality control samples taken and tested 	by the 	Contractor.  One copy of each completed form should be retained by the Contractor until the work is completed and accepted. The original A signed copy of the test data is to be delivered to the District Materials Supervisor. To be an effective quality control function, tests must be completed and results distributed in a regular and timely manner. The plan, therefore, must state what action will be taken in the event that testing and reporting are not completed in a reasonable period of time - preferably within72 hours after the sample is taken.

Control Charts: The specifications require the plotting of gradation test results on control charts using the moving average concept as described in MP 300.00.51. The QC Plan should state where and how the charts will be maintained and made available to Division personnel. These charts are part of the Division's acceptance procedures and must be available to the Division when the project is completed or at the request of the Division personnel. At the contractor's request, the requirement of Control Charts may be waived on a per project Basis. The Contractor will submit a written request to the Division asking that the Control Charts be waived. Division will make a determination based on the size of the project and the number of gradation tests required.

Disposition of Non-Specification Material: A detailed plan of action providing for the immediate notification of all parties involved in the event that nonconforming situations are detected.

Types of QC Plans

#### QC Plans which are intended for use on more than one project shall be defined as Master QC Plans.  Section 6.1 outlines the procedures for Master QC Plan submittal and approval.

#### QC Plans which are intended for use on a single project shall be defined as Project Specific QC Plans.  Project Specific QC Plans shall contain a cover letter which includes the following: project description, CID#, and Federal and/or State Project Number.

#### A contractor may submit a Master QC Plan instead of a Project Specific QC Plan.

#### Once any QC Plan is approved for a project, the key date shall be entered in Site Manager by the appropriate District Materials personnel.   The first date entered shall be the date the Project QC Plan letter is received.  The second date shall be when the district approves the QC Plan for use on the project.

# MASTER QUALITY CONTROL PLAN

The intent of a Master QC Plan is to facilitate the approval process in a more uniform manner.  Master QC Plans can be submitted to the Division by the Contractor when their work load in a given District is routinely repetitive for the year.

The Contractor shall submit a Master Aggregate Items QC Plan yearly to each District in which they have work (see Attachment #2). If the Contractor does not have work in a given District for the year, then a Master Field QC Plan shall not be submitted to that District.

The District will review the submitted Master QC Plans to see if they meet the requirements for an Aggregate Items QC Plan as per section 5.3 and assign a laboratory reference number to the Master QC Plan upon approval for future referencing. The District will acknowledge approval of each Master QC Plan to the Contractor by letter (see Attachment #3), which will include the laboratory reference number and a copy of the approved Master QC Plan. This will then be scanned and placed in ProjectWise under the appropriate District’s Org for that Contractor and/or Producer/Supplier.

Once a project has been awarded, if a Contractor elects to use the approved Master Aggregate Items QC Plan on that project, the Contractor shall submit a letter requesting to use the Master QC Plan for that project.  This letter must be on the Contractor’s letterhead paper, be addressed to the District Engineer/Manager or their designee, and contain the following information: project number, CID#, project description, type of QC Plan, and the laboratory reference number for the Master QC Plan.  (See attachment #4.)

The District shall review the referenced Master QC Plan to ensure it covers all items in the project.  If the referenced Master QC Plan is found to be insufficient for some items on the project, the District shall request the Contractor to submit additional information for quality control of those items as an addendum on a project specific basis.  When the District is satisfied with the QC Plan for this project, a letter shall be sent to the Contractor acknowledging approval (see Attachment #5), with the following attached: the contractor’s project QC Plan request letter and the Master QCP approval letter. This shall then be placed in the project’s incoming-mail mailbox in ProjectWise.

A Master QC Plan that has been approved for project use shall be good for the duration of that project.

For the use of Division Personnel, the District approval letter for this project must state the ProjectWise link to the referenced Master QC Plan for that Contractor.  For example, WVDOT ORGS > District Organization #> Materials > Year>Master QC Plans, etc.

The Master Aggregate items QC Plan shall be valid for the duration of one calendar year beginning on January 1st and ending on December 31st.

# Acceptance Plan

The specifications state that acceptance (verification) sampling and testing is the responsibility of the Division. Quality Control tests are the responsibility of the Contractor. Acceptance activities (sampled and tested at the frequency given in Section 7.1.2) may be accomplished by conducting verification sampling and testing completely independent of the Contractor and, in some cases, by witnessing tests performed by the Contractor, or by a combination of the two. The following guidelines provide a system which should result in sufficient confidence in the Contractor's documentation of his quality control operations to permit acceptance of the material in accordance with the procedure set forth in the specifications.

Review all information supplied by the Contractor on the Quality Control Plan. Note in particular the qualifications of the sampler, tester, the location, and other qualifying statements about the testing facility. In the event the testing facility is such that little qualifying information is supplied or known, this facility should be visited prior to the work and reviewed relative to the availability, type, and suitability (including applicable calibration checks) of the testing equipment. This information should be documented and kept available at the District Materials Section.

Sample and test for applicable items completely independent of the Contractor at a frequency equal to or greater than ten (10) percent of the frequency for testing given in the approved Quality Control Plan. Witnessing the Contractor's sampling and testing activities may also be a part of the acceptance procedure, but only to the extent that such tests are considered "in addition to" the ten (10) percent independent tests.

Plot the results of gradation tests performed by the Division on the Contractor's quality control charts with a red circle, but do not include these values in the moving average. When the Contractor's tests are witnessed, circle the Contractor's test result on the control chart with red.   These values are used in the moving average calculations. The laboratory number will always start with an "M" for all acceptance (verification) samples taken and tested in this manner by the Division, and will always start with a "0" for all of the Contractor's tests which are witnessed by the Division.

Evaluate the results of acceptance (verification) tests, whether performed or witnessed by the Division, in accordance with MP 700.00.54.

If the evaluation indicates similarity with the quality control test, the control chart will be considered acceptable to that poi nt.

If dissimilarity is determined, an immediate investigation will be conducted in an effort to determine the cause. Until the situation is resolved, any samples held in accordance with ML 25 will be retained and may be used in whatever manner deemed appropriate during the investigation.

Implement ML-25 for aggregate gradations.

____________________________

Ronald L. Stanevich, P.E.

Director

Materials Control, Soils and Testing Division

RLS: Mbe

ATTACHMENTS