Mr./Ms/Mrs ________________________

West Virginia Division of Highways

District: ____Engineer/Manager

_______________,West Virginia

Dear Mr/Ms./Mrs

Subject:	Asphalt Concrete  Yearly Master Quality Control Plan for Plant Operations

We are submitting our asphalt concrete hot-mix asphalt Quality Control Plan, developed in accordance with Section 401 of the _________________ Standard Specifications, the

________________ Supplemental Specifications, MP 401.03.50 and the _______________Special Provisions.

- Make of Plant 	Type 	Location

________________________________________________________________________________

- The Quality Control program is under the direction of ____________ to be contacted at Plant/Office in person, by telephone  #  and /or email address:________________

- Sampling and testing will be performed by qualified personnel as per WVDOH Specification Section 106 Control of Materials.

,

- The types of asphalt paving materials to be produced and controlled are as

follows:

  - _____________

  - _____________

  - _____________

  - _____________

  - _____________

  - _____________

- Prior to production of the items, we will submit (on Division Form T-400) our plant mix formula for each type of mix. Only approved materials will be incorporated in the mix.

- During the production operations of the asphalt concrete we will perform at a minimum

Quality Control tests in accordance with the attached schedule.

- All testing  and evaluation  will  be  completed within  24  hours  of  sampling and all documentation will be completed and submitted to the Division on approved processing forms within 72 hours or production will be halted until these items are current.

- Material found to be noncomplying shall not be incorporated into the roadway.  In the event that non-specification material is incorporated into the project, the Division of Highways District Materials Supervisor will be notified immediately.

- We will notify all appropriate Division of Highways personnel at least 24 hours before the scheduled work is to begin.

- (Statement of disposition of nonconforming material)

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

Very truly yours,

Company Representative

GUIDE FOR QUALITY CONTROL PLANS FOR ALL ASPHALT CONCRETE

| TEST OR ACTION | FREQUENCY | TEST METHOD | METHOD OF DOCUMENTATION |
| --- | --- | --- | --- |
| Construction of stockpile to prevent segregation intermingling | Constant | Visual | Diary |
| Coarse aggregate unit weight | One test before start of operation | AASHTO T19 | T304 |
| Stockpile & cold bin gradations | Plant setup and as needed to control production | AASHTO T27 and T11 | T300 |
| Calculate % aggregate from each bin, calibration cold bin | Plant setup |  | T415 |
| Check feeder gate output at gate setting to be used | Plant setup. If bins overflow or run dry. |  | Plant Inspection Form and Diary |
| Select screen sizes | Plant setup |  | Plant Inspection Form |
| Determine hot bin gradation, calculate combined gradations | Weekly during production | AASHTO T27 and T11 | T300 and T415 |
| Calibrate hot bins, select gate openings, and calculate batch weights | Plant setup and change in material source |  | Plant Inspection Form and Diary |
| Check accuracy of scales | Plant setup and weekly accuracy checks. Zero balance and sensitivity each ½ day of operation | Construction Manual – 700 Series | Plant diary and T603 |
| Calibrate asphalt pump, calculate settings | Plant setup |  | Plant Inspection Form |
| Check metering pump at setting to be used | Plant setup and monthly |  | Plant Inspection Form and Diary |
| Reset metering pump to compensate for temperature change | Plant setup and each temperature change of 10 °F (6 °C) |  | Plant Inspection Form and Diary |
| Adequate heated storage for liquid asphalt | Plant setup |  | Plant Inspection Form |

GUIDE FOR QUALITY CONTROL PLANS FOR MARSHALL DESIGNED ASPHALT CONCRETE

| TEST OR ACTION | FREQUENCY | TEST METHOD | METHOD OF DOCUMENTATION |
| --- | --- | --- | --- |
| Calculating mixing time | Plant setup and when paddle pitch or dam gate changed |  | Plant Inspection Form and Diary |
| Ross Count (degree of coating) | Only if mixing time is less than 45 seconds | AASHTO T195 | Diary |
| Coarse aggregate face fracture (Gravel only) | One test before start of operation Every 10,000 ton (9,000 Mg) thereafter | MP 703.00.21 | T302 |
| Complete mix face fracture (When using gravel) | One per week | MP 703.00.21 | T302 |
| Check moisture content of aggregate | Plant setup and daily |  | Diary |
| Temperature check | Minimum of one check of mix per hour at plant |  | Plant Control Chart and Diary |
| Asphalt Content | In accordance with WVDOH MP 401.02.27 | AASHTO T308 (Method A) | T417, T423 and Control Charts |
| Aggregate Gradation (cold feed, hot bins, or completed mix) |  | AASHTO T27 plus T11 or AASHTO T30 | T300, T404, or T417 Plus T425 |
| Daily Mix Property Testing: Stability and Flow, % Air Voids, and % Voids-in- Mineral Aggregate (VMA) |  | AASHTO T245 or ASTM D5581, AASHTO T269, T166, T209, and MS-2 Manual | T406 and T423 |

GUIDE FOR QUALITY CONTROL PLANS FOR

SUPERPAVE DESIGNED ASPHALT CONCRETE

| TEST OR ACTION | FREQUENCY | TEST METHOD | METHOD OF DOCUMENTATION |
| --- | --- | --- | --- |
| Calculating mixing time | Plant setup and when paddle pitch or dam gate changed |  | Plant Inspection Form and Diary |
| Ross Count (degree of coating) | Only if mixing time is less than 45 seconds | AASHTO T195 | Diary |
| Coarse aggregate face fracture (Gravel only) | One test before start of operation Every 10,000 ton (9,000 Mg) thereafter | ASTM D5821 | T302 |
| Complete mix face fracture (When using gravel) | One per week | ASTM D5821 | T302 |
| Check moisture content of aggregate | Plant setup and daily |  | Diary |
| Temperature check | Minimum of one check of mix per hour at plant |  | Plant Control Chart and Diary |
| Gyratory Compaction | In accordance with WVDOH MP 401.02.29 | AASHTO TP4 | T419 |
| Aggregate Gradation |  | AASHTO T30 | T417 and T425 |
| Asphalt Content |  | AASHTO T308 (Method A) | T417 and Control Charts |
| Percent Air Voids |  | AASHTO T166, T209, and T269 | T419 and Control Charts |
| Percent Voids in Mineral Aggregate (VMA) |  | AASHTO PP-28 |  |
| Percent Voids Filled With Asphalt (VFA) |  | AASHTO PP-28 |  |

Mr./Ms/Mrs ________________________

West Virginia Division of Highways

District: ____Engineer/Manager

_______________,West Virginia

Subject:	Asphalt Concrete Yearly Master Quality Control Plan for Field Operations

Dear Mr./Ms/Mrs. ___________________

We are submitting our asphalt concrete hot-mix asphalt Quality Control Plan, developed in accordance with Section 401 of the _________________ Standard Specifications, the ________________ Supplemental Specifications, MP 401.03.50 and the _______________ Special Provisions.

- The field operation is under the direction of ____________ by telephone  #  and /or email address:________________

- will be responsible for insuring that all items of work will comply with Division specifications.

- During the placement operation of the asphalt concrete pavement we will perform, at a minimum, Quality Control tests as per the attached schedule.  Sampling and testing will be performed by qualified personnel as per WVDOH Specification Section 106 Control of Materials.

- All sampling and testing will be completed within the time limits specified by the Division

or work will be halted.

- Material found to be non-complying shall not be incorporated into the roadway.  In the event that non-specification material is incorporated into the project, the Division representative will be notified immediately.

- We will notify all appropriate Division  personnel at  least  24  hours  before work  is scheduled to begin.

Very truly yours,

Company Representative

GUIDE FOR QUALITY CONTROL PLANS FOR FIELD

| TEST OR ACTION | FREQUENCY | TEST METHOD | METHOD OF DOCUMENTATION |
| --- | --- | --- | --- |
| Temperature of mix | 1 per hour | Section 401 of Standard Specifications | Diary |
| Temperature of base | 1 per hour | Section 401 of Standard Specifications | Diary |
| Temperature of mat | 1 test per hour of placement | Section 401 of Standard Specifications | T401 |
| Density | 5 tests per 1000 feet (300 meters) of paving width or rollerpass when applicable. | Section 401 of Standard Specifications | T401 or T407 |
| Tack/Prime Application Rate | Each load or per ½ day of operation whichever occurs first | Section 408/409 of Standard Specifications | Diary |
| Calibration of Nuclear Gauge | As per MP 717.04.21 | As per MP 717.04.21 | Factory Data Sheet |
| Distribution of Test Data | Within 24 hours of completion of testing of a Lot | As per MP 717.04.21 | As per MP 717.04.21 |

Mr/./Ms/Mrs.  _______________________

WV Department of Highways

District  ___   Engineer

__________,  WV

RE:	Asphalt Concrete Quality Control plan for Plant - Project

Fed. Project No.  ___________________________

State Project No. ___________________________

Contract ID No.    __________________________

County :	_____________________________

Description	_____________________________

Dear Mr./Ms/Mrs. _____________________,

We would like to use our approved master quality control plan, reference number ______________ for the project referenced above.

The Quality control plan is under the direction of ___________________________________,  ________________(title), and will be the company’s contact representative to the Department of Highways  district materials and construction departments.  He/She can be contacted in person at the plant, by telephone ________________________________ or at email account _________________

Very Truly yours,

Company Representative

EXAMPLE

Mr./Ms/Mrs  _______________________

WV Department of Highways

District  ___   Engineer

__________, WV

RE:	Asphalt Concrete Quality Control plan for Field - Project

Fed. Project No _____________________________

State Project No.     __________________________

Contract ID No.      __________________________

County :	  _____________________________

Description	  _____________________________

Dear Mr./Ms/Mrs. _____________________,

We would like to use our approved Master Quality Control Plan,   reference number ______________ for the project referenced above.

The Quality control plan is under the direction of ___________________________________,  ________________(title),  and will be the company’s contact representative to the Department of Highways  district materials and construction departments.  He/She can be contacted in person at the project,  by telephone ________________________________ or at email account _________________.

Very Truly yours,

______________________

Company Representative